Dynamic rheological study on o/w emulsions as affected by polysaccharide interactions using a mixture design approach

I Nor Hayati1, CP Tan2, YB Che Man2 and I Nor Aini3

1Food Science Department, Faculty of Agrotechnology & Food Science, Universiti Malaysia Terengganu, 21030 Kuala Terengganu, Terengganu, Malaysia

2Food Technology Department,  Faculty of Food Science & Technology, Universiti Putra Malaysia, 43400 Serdang, Selangor, Malaysia

3Product Development & Advisory Division, Malaysian Palm Oil Board, 43000 Kajang, Selangor, Malaysia

Abstract

Interaction effects of xanthan gum (XG), carboxymethyl cellulose (CMC), and locust bean gum (LBG) on the viscoelastic properties of oil-in-water emulsions were investigated by mixture design experiments.  Blends of XG, CMC and LBG were prepared according to an augmented simplex-centroid mixture design (10 points) and used at 0.5% (wt/wt) in the emulsion formulations. An appropriate mathematical model was fitted to express each response as a function of the proportions of the blend components that are able to empirically predict the response to any blend of combination of the components. The synergistic interaction effect of the ternary XG:CMC:LBG blends at approximately 33-67% XG levels was shown to be much stronger than that of the binary XG:LBG blend at 50% XG level (p < 0.05). Nevertheless, an antagonistic interaction effect became significant as CMC level in blends was more than 33% (p < 0.05). Elastic modulus and loss tangent responses were successfully fitted with a special quartic model while viscous modulus was fitted with a full quartic model (R2adjusted  > 0.98). This study found that a mixture design approach could serve as a valuable tool in better elucidating and predicting the interaction effects beyond the conventional two-component blends.