Opportunity of pulsed electric fields technology (PEF) to fabricate dysphagia friendly chickpea flour-based gels

F Drudi1, J King2,3, S Leong2,3, I Oey2,3, U Tylewicz1,4

1Department of Agricultural and Food Sciences (DISTAL), University of Bologna, Piazza Goidanich 60, 47521, Cesena, Italy

2Interdepartmental Centre for Industrial Agri-Food Research (CIRI), University of Bologna, Via Quinto Bucci 336, 47521, Cesena, Italy

3Department of Food Science, University of Otago, PO BOX 56, Dunedin, 9054, New Zealand

4Riddet Institute, Private Bag 11 222, Palmerston North, 4442, New Zealand

The ability of chickpea (Cicer arietinum) flour slurry (10% w/w) in forming dysphagia friendly gel under the influence of pulsed electric field (PEF) technology applied at 1-2 kV/cm electric field strengths and 410-500 kJ/kg specific energies was investigated in this study. Results showed that PEF treatment of chickpea slurry with specific energies above 410 kJ/kg was necessary to form soft gels after overnight cooling (24 h, 4°C). Hardness and rheological properties such as storage modulus and yield stress of the gel further increased with energy applied (from 0.35±0.01 to 2.94±0.19 kPa, from 195±42 to 2350±466 Pa and from 15.7±5.6 to 423.4±135.2 Pa, respectively), but the tested electric field showed no significant impact on gel characteristics. Notably, gels formed after higher field strengths application exhibited increased readily digestible starch (from 16.28±0.15 to 89.06±1.78 %) and a faster intestinal protein digestion rate (from 1.16±0.20 ·10-2 to 1.65±0.04 ·10-2 min-1) during simulated gastrointestinal digestion. Findings from FTIR, DSC and light microscopy collectively revealed that gel formation was mainly attributed to starch gelatinization caused by a temperature increase due to Joule effect during PEF, while protein denaturation and aggregation became prominent in chickpea slurries after PEF-treated above 450 kJ/kg, resulting in a more solid-like gel formation. Overall, PEF technology has proven to be an effective and rapid method (44.4-227.4 ms) for producing chickpea gels with varying textural and rheological consistencies from a single ingredient, which can be particularly applicable for patients with different degrees of dysphagia, as highlighted by the wide range of IDDSI (International Dysphagia Diet Standardization Initiative) classification levels achieved with chickpea gels formed after PEF pre-treatment.